ECP2046 COMPUTER ORGANIZATION AND ARCHITECTURE

SESSION 2001/2002

SOLUTION FOR TUTORIAL-4

Q1.

A subroutine is called by a program instruction to perform a function needed by a calling program. An interrupt service routine is initiated by an event such as an input operation or a hardware error. The function it performs may not be related to the program being executed at the time of interruption.

Q2.

- Interrupts should be enabled, except when C is being serviced. The nesting rules can be enforced by manipulating the interrupt-enable flags in the interfaces of A and B.

- A and B should be connected to INTR2 and C to INTR1. When an interrupt request is received from either A or B, interrupts from other devices will be automatically disabled until the request has been serviced.

Q3.

Caches depend on two kinds of address locality to achieve their goal :

Spatial locality - the observation that memory locations with addresses numerically similar to a recently accessed memory locations are likely to be accessed in the near future.  Caches exploit this property by bringing in more data than have been requested, with the expectation that future requests can be anticipated.

Temporal locality - occurs when recently accessed memory locations are accessed again.  This may occur, for example, to memory locations near the top of the stack, or instructions inside a loop.  Temporal locality is exploited in cache designs primarily by the choice of what to discard on a cache miss.  Many cache replacement algorithms exploit temporal locality by discarding those entries that have not been recently accessed.

Q4.

Main memory size = 24+4+8= 216= 64 K

Size of the block   = 2byte=28= 256

Number of sets     = 2set = 24= 16

Q5.

- 4096 blocks of 128 words each require 12 + 7 = 19 bits for the main memory address.

- TAG field     = 8 bits

SET field      = 4 bits

WORD field = 7 bits

Q6.

DMA will be serviced first. Because every clock cycle is a break point for the DMA to be acknowledged where as interrupts have break points only at the end of each instruction cycle.

That is the CPU will respond to interrupts only after the completion of the current instruction but the DMA will be acknowledged for every clock cycle.