Important formulas

- To find the work function of the material

 is the work function  of a metal =        = h     =     = hc /  o

2.  The kinetic energy of the electron,

KE = [(hc) / (e o)]  -  

3. To find  out the  photoelectric current density, which is the charge  flux,

J = eI  / 

## J =  eI / hc

4. De-Broglie wavelength      =   h/mv  =  h / p

5. Heisenberg uncertainity for position and momentum  Px . x >= h /2

Heisenberg uncertainity for Energy and time  E . t >= h / 2

Minimum uncertainity in the velocity is =   vx =    Px /  m

6. To calculate the fermi energy

First u calculate the concentration of the metal

n =  d NA / Mat   ( NA is the Avagadro’s Number = 6.023X1023)

The fermi energy at 0K is given by

EFO  =  (h2 / 8 me) ( 3n/ ) 2/3

The average velocity ve of the conduction electron ve = (6 EFO  / 5 me)1/2

7. The equilibrium  hole concentration po=  ni2 / no

The equilibrium  electron  concentration no=  ni2 / po

### The relation of electron and hole  concentration  with fermi energy is

n0=nie(EF−Ei)kT

po = ni e(EF -  Ei) / kT

EF -  Ei = kT ln (no / ni)  for electrons

EF -  Ei = kT ln (po / ni)  for holes

8.